| ACTIVITY & LOCATION | HAZARDS | RISK | PERSONS AFFECTED | INITIAL RISK RATING (before controls in place) | Control Measures | RESIDUAL RISK RATING (after controls in place) | PERSONS RESPONSIBLE (for implementing control measures) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NOTE: Block B houses RCSI Depts. Block C leased out to a 3rd party. Damer Hall used by RCSI for teaching, sports classes and events. |  |  |  |  |  |  |  |
| Manual Handling tasks throughout the buildings. | Lifting, putting down, pushing, pulling, carrying and moving | Lower back injuries, upper limb injuries, foot injuries and musculoskeletal injuries. | Security, Porters, Media Services, Contractors and anyone else in the building who carry out these activities | 3; 3; 9. | On site staff i.e. Security and Porters are trained in safe manual handling practices. Others who bring in heavy or bulky loads are advised to ensure that their own employees are suitably trained in safe manual handling practices. Only lift and carry what you are capable of. If heavy, do not attempt. Get help. Trolleys are available for heavy or awkward loads. Passenger lifts can also be used to bring equipment to upper and lower floors. To always check the maximum load the lift can take including the equipment and people in it. Access into the buildings vary i.e. Block B – from street level at front of building and RH side of building. From both entries, there are a small number of steps up onto the lift lobby. From the side entrance there is an access lift for those who need it and for deliveries etc. Block C – access via steps from SSG side or via lift next to steps or access via Cuffe Lane via ramp. Damer Hall – from street level down steps or from Cuffe Lane via ramp into courtyard. | 2; 3; 6. | Line Managers of staff and others carrying out the tasks. Staff and others to ensure they implement the safe principles of Manual Handling. |
| Walking and moving throughout the building. | Wet floors, uneven or upturned flooring, trailing cables, bags on floors, staircases, steps. | Slips, trips and falls. | All Staff, Students, Contractors and Visitors in the building. | 3; 3; 9. | Block B, C and Damer Hall are maintained in good condition with adequate lighting levels throughout. Cleaners on standby during opening hours in Block B & Damer Hall to clean up any spills asap. Cleaners will place signage where floors may be damp after washing. Washing of floors etc. will take place after hours. Trailing cables where required must be taped down or moved away from access ways. Any damage to flooring etc. to be notified to Security at Reception or estate@rcsi.ie / 01 402 2101 as soon as possible. | 2; 3; 6. | Estate & Support Services – Cleaners, Security. Contractors. All staff and students to report to Estates/Security any slip, trip and fall hazards spotted. |
| General building activities, contractors carrying out maintenance works, kitchen works. | Electrical Equipment, Hot works, poor housekeeping. | Fire, explosion, burns, alarm activation, sprinkler activation. | All Staff, Students, Contractors and Visitors in the building. | 4; 5; 20. | Planned preventative maintenance organised for all fire activation equipment regularly. Good housekeeping is promoted to ensure that means of escape routes and fire doors are kept clear at all times. Fire evacuation drills are carried out in line with RCSI emergency procedures and best practice requirements. Contractor carrying out hot work are subject to a permit to work system, where works are organised in advance and alternative fire emergency procedures are put in place if required. Fixed electrical testing and PAT testing carried out on all electrical equipment when applicable and as required. | 2; 5; 10. | Estate & Support Services – testing of all electrical and firefighting equipment and managing contractors hot works. All staff to practice good housekeeping. |
| Medical assistance following an injury | Lack of first aiders, first aid equipment, slow response | Injury / medical condition deteriorates | All Staff, Students, Contractors and Visitors in the building | 2; 4; 8. | All staff to follow the injury, incident and near miss reporting procedure. RCSI’s Security Team are the First Responders for all emergencies in RCSI including first aid response. Contact RCSI Security immediately for first aid assistance on 191 from any RCSI landline phone or 01 402 2219 from a mobile or other phone. The first aid bag in Block B is located at the Reception desk on ground floor with Security. A defibrillator in Block B is located at reception on the ground floor. In Block B there is 1No. evacuation chair, located on 4th floor in stair well. This can be used to assist mobility impaired persons from the building in the event of an emergency. Security and Porters are trained in evacuation chair use. Only call an ambulance if it’s an emergency situation and hospital assistance is required. RCSI Security will liaise with the ambulance service otherwise if required. | 1; 4; 4. | RCSI Staff and Security |
| Reception areas. | Unauthorised access to the building | Violence and aggression from others | Reception and Security staff | 3; 3; 9. | RCSI Security are all PSA approved and trained to deal with security situations that may arise in the building. All RCSI Security Staff carry radios which are also linked into the Estate and Support Services Department. All personnel who plan to enter the buildings will need to swipe to access. Visitors will report to Security in Block B Reception and sign in/out of the building. | 2; 2; 6. | RCSI Security and Estate and Support Services Department |
| Lone working or out of hours work | Lack of help or support during an incident | Incident or injuries deteriorate | All Staff, Students, Contractors in the building after hours and or working alone. | 3; 4; 12. | Staff to follow the lone working / out of hours procedure. Staff to complete the out of hours/lone working form in advance of the work and forward to RCSI Security and the Health & Safety Office to seek approval to work during these times. If high risk work is taking place (see list in procedure) a buddy system needs to be implemented. Contractors work will be arranged via the Estate & Support Services Dept. Work will only take place out of hours if absolutely necessary. | 2; 4; 8. | All RCSI Staff Estate & Support Services – Contractor work |
| Access to high shelving units or presses Using steps / ladders | Working at height | Falling from height | Staff, Students, Contractors. | 3; 4; 12. | Avoid if at all possible accessing higher shelving and working at height. If unavoidable, use only suitable stepladders, which are tagged safe to use to access higher shelving if necessary. Ensure you have a colleague / fellow student to help you obtain the equipment / materials. Contractors and RCSI maintenance only to use suitable class 1 maintenance ladders, which are tagged safe to use for working at height. | 2; 4; 8. | All RCSI Staff and Students Contractors |
| Office work | Working with display screen equipment | Lower back pain, upper body strain, eye strain | Staff | 4; 3; 12. | Click here for details on how to set up your workstation safely. Further details on set up also contained in the Health & Safety Induction course for Staff. | 2; 3; 6. | Staff |
| Work / access to roofs. | Working at height | Falling from height | Staff, Students, Contractors | 4; 5; 20. | There is strictly no access to any roof in the Ardilaun Buildings for any staff or students at any time. Access procedures are in place for maintenance only. RCSI Contractors and maintenance staff will only be allowed access to roofs once they liaise with the Estate and Support Services Dept. and the Permit to Work system is implemented. All roofs have handrail system in place acting as a barrier to prevent falls. | 1; 5; 5. | Staff, Students, Contractors, Estate & Support Services. |
| Window cleaning internally and externally | Working at height, use of cleaning equipment, trailing cables, working with chemicals | Falls from height, slips, trip, falls, and direct contact of chemicals with the skin. | Contractors Staff, Students, Visitors | 4; 4; 16. | Windows will be cleaned by RCSI in Block B only. Block C windows will be cleaned by the tenant. Competent contractor will be used. Method statement and risk assessment provided by contractor before work carried out and reviewed and approved by the Health & Safety Office. Window cleaning carried out “out of hours”. Where local cleaning occurs during the day, the area will be cordoned off to prevent staff, students and others entering. | 2; 4; 8. | Window cleaning contractor |
| Access / egress for deliveries, waste collection etc. - pick up and drop off | Vehicle and pedestrian traffic, barrier | Hit / struck by moving vehicles or barrier | RCSI Staff – Porters, Maintenance etc. and contractors - delivery / collection personnel | 2; 5; 10. | RCSI Security will open barrier to let vehicles through. Once barrier is opened you have access to the Ardilaun Courtyard. The barrier has a sensor in place where if any obstruction underneath while closing it will automatically stop. Preventative maintenance on the barrier takes place every 6 months. | 1; 5; 5. | RCSI Staff – Porters, Maintenance etc. and contractors - delivery / collection personnel and Estate and Support Services. |
| Oil tanks behind Block C. | 2No. manholes to oil tanks behind block c. | Fall from height | RCSI Staff, Contractors | 3; 4; 12. | This oil tanks is no longer in use. Has not being used since RCSI took over these buildings. There is no oil in the tanks. Manhole cover are in place and secure. Tanks themselves are not very deep; however access is a street level. If you access form back of block C – this is lower street level. There is no need to access these tanks for any reason by any contractor or staff member. | 2; 4; 8. | RCSI Staff – Porters, Maintenance etc. and contractors personnel and Estate and Support Services. |

Risk Ratings

LIKELIHOOD CATEGORIES

| Category | Definition |
| --- | --- |
| 1 | Practically Impossible |
| 2 | Not Likely |
| 3 | Possible |
| 4 | Likely |
| 5 | Very Likely |

| Category | Definitions |
| --- | --- |
| 1 | First Aid, Near Miss |
| 2 | Lost Time Accident (LTA) (1 Day), Medical Treatment |
| 3 | LTA (3 Day-Reportable), Dangerous Occurrence |
| 4 | Single Fatality |
| 5 | Multiple Fatality |

CONSEQUENCE CATEGORIES

Risk Rating Matrix